# Smart home model

We will follow this site: https://www.student.unsw.edu.au/thesis-structure & provided presentation

# Abstract

- Table of Contents

1	Abstract	2

Table of Figures	5

2	Introduction	6

3	Methods	7

3.1	Circuit design	7

3.1.1	Control panel PCB	7

3.1.2	Main PCB	7

3.2	3D-Modelling	8

3.3	Assembly of the Model	9

3.4	Problems during assembly	10

3.5	Introduction to software	10

3.6	Room 1 software	12

3.7	Room 2 software	13

3.8	Room 3 software	13

3.9	Door software	14

3.10	Control panel software	15

3.10.1	Menu implementation	16

3.10.2	Display implementation	18

3.10.3	Mediator implementation	18

3.11	Software problems	19

4	Results	21

5	Discussion	22

6	Conclusion	23

7	References	24

8	Appendices	25

# Table of Figures

Figure 1 Control panel PCB 3D model	7

Figure 2 Main PCB 3D model	8

Figure 3 Base 3D model	8

Figure 4 Walls and floor model	9

Figure 5 Main program	11

Figure 6  Software diagram	11

Figure 7 Check button function	12

Figure 8 Room1.h	12

Figure 9 Update room 1 function	13

Figure 10 Update room 2 function	13

Figure 11 Room3.h	14

Figure 12 Update door function	15

Figure 13 Init control panel function	15

Figure 14 Part of the update control panel function	16

Figure 15 Menu structs	17

Figure 16 Menu diagram	18

Figure 17 Display.h	18

Figure 18 Mediator.h	19

Figure 19 get_getter_func function	19

# Introduction

States the objective or purpose of the experiment and provides the reader with important background and/or theory to the experiment/project

- What kind of problem did you work on?

- Why did you work on this problem?

- What should the reader know or understand when he/she is finished reading the report?

# Methods

## Circuit design

### Control panel PCB

We chose to have a Control panel, because a characteristic of a smart home is according to, (Alam, Reaz, & Ali, 2012) that it can be controlled remotely from outside the home. The control panel could be developed further to be wireless with Bluetooth and controlled via an app.

For the Control panel, we chose to use buttons to navigate in a menu displayed on the LCD.

A Potentiometer was planned to control the RGB Values and the brightness of the LEDs.

We incorporated 5 buttons UP, DOWN, OK, BACK and a Button for an extra function (sleep mode).

The 4k7 Ω Resistors function as Pull up Resistors for the I2C Module of the LCD.

Figure 1 Control panel PCB 3D model

### Main PCB

We decided to have the components in and on the house (switch-buttons, LED, Potentiometer, Sensors) connected to the PCB with pin header connectors. So, the PCB is detachable from the House. We Further divided the circuit in two PCB’s. One for the Control panel, with the Buttons and Potentiometer mounted on the PCB via Through hole soldering and a pin header connection to the Liquid Crystal Display (LCD).

The other PCB has the transistor driver circuit for the LED's, circuits for the Sensors and the microcontroller on It.

Figure 2 Main PCB 3D model

## 3D-Modelling

Design of the house can be divided into two categories, one is base and the other are walls and floor.

Base was designed in Fusion 360 for 3D printer, whole model consists of 230mm X 230mm square with three rooms. We also added mounting holes for walls and floor and space under the floor is empty for routing cables, for that reason we also added openings at the back near the control panel.

Figure 3 Base 3D model

Walls and floor were designed in Fusion 360 as well, but they were designed for a laser cutter. After laser cutting some parts had to be treated using a rasp so they fit together, because some connections are angled 45 degrees for stronger fit.

Figure 4 Walls and floor model

For slicing we used Bambu Studio and printed it as well with a Bambu 3D printer.

## Assembly of the Model

After receiving the drilled PCBs, we checked the connections with a multimeter.

Because the PCBs had no connection errors, we continued with the next step: soldering the PCBs.

To have more space underneath the PCB when placing it on the table, we first soldered some of the pin headers to make it stand stable while soldering the vias.

Then we soldered the components starting with the flattest and ending with the tallest.

We reused the female-to-female jumper cables to create the wiring to the components in the house. So, the cables were cut in half and soldered to flex wire. (insert average length) Then the flex wires were soldered to the components according to the color-coded wiring table. We used shrinkable tubes to isolate the soldered wire to prevent short circuits. Now with the wiring the entire system and code could be tested.

The Control Panel PCB was mounted with 3 x mm screws and 12 mm long spacers on to the Mainboard.

Where the LEDs should go, 5mm holes got drilled in the walls for the Lens to go through. To mount the LDR sensor and the PIR sensor, holes were drilled in the outer walls as well.

To arrange the LEDs, buttons and sensors in the house, it was marked where the wires go through the floor. According to that, the floor plates got holes drilled through the wood.

With the components placed on the walls, the female connectors were thread through the holes in the floor, routed at the bottom and lead to the casing of the Control Panel.

With all the connections according to the wiring table, the casing of the control panel could be closed and the walls screwed to the bottom with 2.5 x mm screws. To fix the wires on the walls we used electrical tape.

## Problems during assembly

On the Main PCB connection issues and other connection issues were discovered while testing the connections with a multimeter. We fixed the problem with resoldering the visual faulty solder joints.

We realized that D0 was just connected on top layer on the Main PCB Layout, a via was forgotten to be placed in front.

Fortunately, it was not relevant for the main functionality because it was code related:

The Wire for Room 1 Button was swapped with Room 1 Pot, because the IO Pin A6 formerly assigned to Room 1 Button can only be used analog.

The Potentiometer on the control panel was not implemented because it was more convenient to code the user interface with only buttons.

The Pin D1/RX was not able to be used because it interfered with serial communication (RX/TX), even without being connected to a PC. That was probably caused by the USB C connector circuit on the microcontroller.

D1 was originally used for button room 2 but then changed to A2.

Because J5 had just one pin connected to A2, the two wires of the buttons in parallel were soldered to one female connector.

## Introduction to software

Our first idea was to create a library for every room, door and control panel. The library should implement “Init()” and “Update()” functions which will be called in main function. In addition to those them we also implemented some functions which control the state of the room directly, for example they turn the light on and off. This approach made code more maintainable and readable, therefore we used it in our final build. Also we could very easily divide individual code parts to team members, in our case everyone had to program one room.

Figure 5 Main program

We had to come up with a solution to how buttons should operate in rooms where there is the possibility to change brightness or color of the light. After a few discussions we stuck to idea last action takes effect. This means that if brightness was set to half of its potential by clicking the button the light is turned off (light is set to maximum brightness after clicking button, only when brightness is zero), even though the potentiometer’s value stays unchanged. This solution turned out to be very handy when we started working on control panel.

Figure 6  Software diagram

When a button is pressed the change from not being pressed and being pressed isn’t immediate, there are always some fluctuations which are result of mechanical construction of a button. We solved this issue by adding delay every time a button is pressed, this resulted in functions for every button which check status of a button and if it is pressed they start 100 ms delay.

Figure 7 Check button function

## Room 1 software

Room 1 has one button for turning the LED on and off and one potentiometer for controlling the brightness of the LED. We implemented functions for controlling and retrieving information about the LED’s status and brightness.

Figure 8 Room1.h

In “Init()” we initialized ADC and PWM using Alin’s libraries, and also set button pin as input with pull up resistor, to do that we referred to AtMega datasheet (Atmel Corporation, 2015) and Arduino Nano pinout (Arduino cc, 2021). These documents we used for other rooms and control panel as well.

“Update()” function checks if potentiometer values changed since last measurement, if yes update LED’s intensity accordingly. Then it checks if button is pressed and wasn’t pressed previously, if this statement is true then the status of LED is inverted.

Figure 9 Update room 1 function

## Room 2 software

This is the simplest room it has only two buttons and one LED. Since we wired buttons in parallel, from the Arduino perspective they act as a one button. Again, we added functions for getting and setting status of the LED. Logic in “Update()” and ”Init()” is very similar to Room 1, just without potentiometer and LED is controlled using registers not using the PWM library.

Figure 10 Update room 2 function

## Room 3 software

Room 3 contains one potentiometer for color changing, a button and one LED. We decided that potentiometer will affect only one property of the light for example red or green.

We were aware of the limitations of this approach and decided to implement configuration of a potentiometer’s-controlled value into control panel, in final version it is possible to choose which part of the light’s color is controlled by a potentiometer. We also added HSV color model because it is much easier to work with for people. So, you can configure red, green, blue, hue, saturation, value whilst red is default.

Figure 11 Room3.h

Potentiometer is read the same way as in the room 2 and LED is controlled using PWM 3 from Alin’s library, turning on and off using the switches between full white and completely off.

## Door software

Door’s LED is controlled by PIR sensor and LDR, therefore there is no button. In addition to “Init()” and “Update()” functions we implemented functions for detecting movement, getting current value of LDR and setting threshold for LDR.

In “Init()” function we just initialized ADC using Alin’s and pin for LED as output and pin for PIR sensor as input using registers.

Every time the movement is detected and LDR has higher value than threshold LED is turned on and timer is reset. Timer is just one variable which is incremented by one every time the “Update()” function is called, when the LED is turned on the variable is set to zero, when this variable reaches particular value LED is turned off.

Figure 12 Update door function

## Control panel software

Even though it is not a room, it works very similarly to one, because buttons function the same as in the rooms and control panel and background logic is just a more complicated version of a LED.

Figure 13 Init control panel function

In addition to physical components, which make up control panel, we had to implement back-end logic for the menu responsible for storing and navigating in menu elements, we also implemented some type of a link or connection between control panel and rooms, we called it mediator.

Figure 14 Part of the update control panel function

Mediator’s responsibilities are to retrieve and change information for room associated with menu element for example for element showing status of LED in room 1, mediator will return status of led in room 1 and element will be updated, when value in menu element is changed mediator ensures that room is updated as well.

### Menu implementation

When we were designing a menu, we took inspiration from files and folders in a computer, every element in a menu can be file or folder.

Folder contains more folders or files, for example at the top level of the menu there are four folders (room 1, room 2, room 3, door) after selecting one of the folders its content will be displayed on control panel. Returning back to previous folder is done by clicking “back” button.

Files hold values which can be linked to room or it can be some arbitrary value which affects other files for example file “Color format” can have value “RGB” or “HSV” and this value affect how is color of the light in room 3 represented on control panel. After selecting file its value can be changed by clicking “up” and “down” buttons, changes take effect instantaneously. File can be deselected by clicking “back” button.

Files and Folders are contained in “Menu_element” struct (W3Schools, C structs, n.d.) this struct has a variable of enumeration type (W3Schools, C enums, n.d.) to determine if current menu element is file or folder, after this variable is union (TutorialsPoint, n.d.) with file or folder struct in it. The only differences between folder and file structure are that file has also value, type of data stored in it and variable which determines if it is selected.

Figure 15 Menu structs

Our first idea for storing menu elements was to use multidimensional array, but we quickly realized that it would be difficult to work with it, because we can’t determine the size of an array in C, and we risk going out of bounds of it. For that reason, we stored all the elements in one dimensional array and location of the element we encoded in its “ID”.

ID acts as a path to the element for example after clicking second folder on first level first element on second level will have ID equals to 12, path is inverted (first level is last digit and last level is first digit) so it is easier to search in array of elements.

Figure 16 Menu diagram

### Display implementation

To simplify the use of display we created helper library, which has functions for updating whole display with menu elements, which are passed to the function as a parameter, and for updating one element, so display is not refreshed every time a value in one element is changed only that element is refreshed.

Figure 17 Display.h

I addition to displaying current menu elements, display also mark hovered element with “*” character and selected element with “>” character.

### Mediator implementation

As it was mentioned in previous chapters to update rooms using control panel we are using Mediator. Mediator’s API consists of two functions which return pointer to a function one returns a function for changing value and the second one returns a function for getting a value. Which function to return is determined by menu element ID, functions aren’t stored in an array pointer to a particular function is returned using switch statement (W3Schools, C switch, n.d.) which switches between different IDs.

Figure 18 Mediator.h

Figure 19 get_getter_func function

## Software problems

When debugging the whole assembly we encountered some issues, those problems were mostly logical and easily fixable. However, we weren't able to fix problems with altering the color of light in room 3 using control panel when color format was set to HSV, therefore this feature is not available and can be changed using control panel only when color format is set to RGB.

Another inconvenience which made its way to the final product is unreliable control of the status of light in door, which means that door light can be controlled by control panel, but it does not work every time, we couldn’t figure out why it happens.

# Results

Present data, state in verbal form as well as visual, use sentences to draw attention to key points in graphs, figures, etc., number and title tables and graphs, and name x/y axes in graphs. All figures, graphs and tables that you include MUST be introduced in the results text, use appendix for raw data or complex calculations

- What are your results?

- Is the data/project outcome presented in a clear, logical and self-explanatory way?

- What is the main point –what ties results/project/delivery outcomes together?

# Discussion

Explain, analyze, interpret, refer back to ALL your initially stated hypotheses, expectations, assumptions in the discussion!

- What do the results indicate clearly?

- What are the sources of error?

- How do the results compare to the theory/hypothesis?

- What is the significance of the results?

- How do you justify that interpretation?

- Suggested improvements for future research?

# Conclusion

Short summary of key results of your research, no new information, it supplements the Abstract

# References

All literature references mentioned here

# Appendices

Apendix/appendices for relevant content